# TASK-2

TITLE: Design a FIR Filter

AIM: To design and simulate a 4-tap Digital FIR (Finite Impulse Response) filter using Verilog HDL and verify its functionality through simulation results.

OBJECTIVE:1. To understand FIR filter architecture.

2. To implement FIR filtering using Verilog HDL.

3. To perform simulation and analyze filter output.

4. To study delay elements, multipliers, and adders used in FIR filters.

SOFTWARE USED:  vivado

THEORY:

A FIR (Finite Impulse Response) Filter is a digital filter whose output depends only on present and previous input samples.

The general FIR filter equation is:

y(n)=k=0N-1hkx(n-k)

Where:

y(n) = Output signal

x(n) = Input signal

h(k) = Filter coefficients

N = Number of taps

In FIR filters:

1.The output becomes stable after finite clock cycles.

2.FIR filters are always stable.

3.They are widely used in DSP, communication systems, and noise reduction applications.

CODE:

VERILOG CODE FIR FILTER :

`timescale 1ns / 1ps

///finite impluse response (FIR) filter

module fir_filter(

input clk,

input reset,

input signed [7:0] x,

output reg signed [15:0] y

);

parameter h0 = 1;

parameter h1 = 2;

parameter h2 = 3;

parameter h3 = 4;

reg signed [7:0] d0, d1, d2, d3;

always @(posedge clk or posedge reset)

begin

if(reset)

begin

d0 <= 0;

d1 <= 0;

d2 <= 0;

d3 <= 0;

y <= 0;

end

else

begin

d3 <= d2;

d2 <= d1;

d1 <= d0;

d0 <= x;

y <= (h0*d0) + (h1*d1) + (h2*d2) + (h3*d3);

end

end

endmodule

TESTBENCH FOR THAT VERILOG CODE:

`timescale 1ns / 1ps

///testbench

module fir_filter_tb;

reg clk;

reg reset;

reg signed [7:0] x;

wire signed [15:0] y;

fir_filter uut(

.clk(clk),

.reset(reset),

.x(x),

.y(y)

);

always #5 clk = ~clk;

initial

begin

clk = 0;

reset = 1;

x = 0;

#10 reset = 0;

#10 x = 1;

#10 x = 2;

#10 x = 3;

#10 x = 4;

#10 x = 5;

#10 x = 6;

#10 x = 7;

#10 x = 8;

#50 $finish;

end

initial

begin

$monitor("Time=%0t Input=%d Output=%d", $time, x, y);

end

endmodule

PRACTICAL OUTPUT:

PERFORMANCE ANALYSIS:

| Parameter | Observation |
| --- | --- |
| Filter type | FIR filter |
| Number of taps | 4 |
| stability | Stable |
| speed | High |
| Hardware complexity | Moderate |
| Output response | Finite duration |

CONCLUSION:

The Digital FIR Filter was successfully designed and simulated using Verilog HDL. The simulation results verified correct filtering operation based on input samples and filter coefficients. The project helped in understanding FIR architecture, delay elements, and DSP-based digital filter implementation